ECE 278C – Imaging Systems

Assignment 3: Phase-Only Image Formation Techniques

Name: Xinyi Yang
Date: October 25, 2025

Part A: Phase-Only Received Signal

1. Implementation

Received wavefield data magnitude set to constant (unit magnitude)

Phase information preserved: s_phase = exp(j·angle(s_original))

Full complex Green's function used for backward propagation

2. Single Source Results

1) Image Characteristics:

Source successfully localized at (0,0) with good spatial accuracy

Reduced sidelobe levels compared to full complex reconstruction

Cleaner point spread function with less background clutter

Maintains excellent peak-to-sidelobe ratio

2) FFT Spectrum Analysis:

Broader frequency distribution compared to full complex case

Phase-only processing emphasizes high-frequency components

Maintains symmetric pattern but with different energy distribution

3. Multiple Sources Results

Source Localization:

All four sources clearly resolved at correct positions

Source 1 at (0,15λ) shows good isolation

Sources 2-4 at y=-9λ maintain proper horizontal spacing

Geometric relationships preserved accurately

4. Observations

1) Advantages:

Excellent source localization capability

Reduced background noise and artifacts

Simplified signal processing (magnitude normalization)

Robust performance across different source configurations

2) Characteristics:

Phase information alone sufficient for accurate spatial localization

Magnitude information primarily affects dynamic range and contrast

Phase-only approach emphasizes coherent interference patterns

Part B: Phase-Only Integration Kernel

1. Implementation

Green's function simplified to phase-only: G_phase = exp(-j·k·R)

Amplitude term 1/√(jλR) removed from kernel

Evaluated with both original and phase-only received signals

2. Phase-Only Kernel with Original Signal

1) Single Source Results:

Good source localization at (0,0)

Different point spread function characteristics

Modified sidelobe structure due to missing amplitude weighting

Slightly broader main lobe compared to full complex

2) Multiple Sources Results:

All sources detectable but with modified intensity relationships

Relative amplitudes between sources altered

Spatial resolution maintained but with different contrast

Increased background levels in some regions

3. Phase-Only Kernel with Phase-Only Signal

1) Single Source Results:

Clean reconstruction with simplified processing chain

Good compromise between Part A and Part B approaches

Reduced computational complexity

Maintains acceptable spatial localization

2) Multiple Sources Results:

All four sources resolvable with correct positioning

Simplified interference patterns

Different intensity relationships between sources

Practical implementation for resource-constrained systems

4. Observations

1) Kernel Simplification Effects:

Removal of amplitude term changes point spread function

Modified depth dependence in reconstruction

Altered sensitivity to source-receiver distances

Different noise characteristics in reconstructed images

2) Computational Benefits:

Reduced mathematical complexity

Elimination of square root operations

Potential for faster implementation in hardware

Simplified numerical processing

Summary

1. Findings

Part A:

Phase-only received signal provides excellent source localization

Maintains spatial resolution while reducing artifacts

Demonstrates phase information sufficiency for imaging tasks

Part B:

Phase-only kernel offers computational simplicity

Combined approach provides practical compromise

Different trade-offs between complexity and quality

2. General Observations:

Phase information is fundamentally more important than magnitude for spatial localization

Phase-only methods enable simplified imaging systems

Application requirements dictate optimal approach selection

Appendix

import numpy as np

import matplotlib.pyplot as plt

from pathlib import Path

from time import perf_counter

class PhaseOnlyImaging:

def __init__(self, lambda_val=1.0):

self.lambda_val = lambda_val

self.k = 2 * np.pi / lambda_val

self.spacing = lambda_val / 4.0

self.x_r = np.arange(-30 * lambda_val, 30 * lambda_val + self.spacing, self.spacing)

self.y_r = -60 * lambda_val

self.x_i = self.x_r

self.y_i = self.x_r

self.fig_dir = Path("assignment3_results")

self.fig_dir.mkdir(parents=True, exist_ok=True)

print(f"Number of receivers: {len(self.x_r)}")

print(f"Image size: {len(self.y_i)} × {len(self.x_i)}")

print(f"Spacing: λ/4 = {self.spacing:.4f}")

def greens_function_2d(self, R):

"""2D Green's function"""

R = np.maximum(R, 1e-12)

return (1.0 / np.sqrt(1j * self.lambda_val * R)) * np.exp(1j * self.k * R)

def simulate_received_signal(self, sources):

"""Generate received signal at receiver array from point sources"""

s_total = np.zeros_like(self.x_r, dtype=np.complex128)

for (x_s, y_s) in sources:

R = np.sqrt((self.x_r - x_s) ** 2 + (self.y_r - y_s) ** 2)

s_total += self.greens_function_2d(R)

return s_total

def phase_only_signal(self, s):

"""Part A: Keep only phase information of received signal"""

magnitude = np.abs(s)

phase = np.angle(s)

return np.exp(1j * phase)

def phase_only_kernel(self, dist):

"""Part B: Phase-only kernel for backward propagation"""

return np.exp(-1j * self.k * dist)

def reconstruct_direct_superposition(self, s, method='full'):

"""Direct superposition with phase-only options"""

reconstructed_image = np.zeros((len(self.y_i), len(self.x_i)), dtype=np.complex128)

for j, y in enumerate(self.y_i):

dx = self.x_i[:, None] - self.x_r[None, :]

dy = y - self.y_r

dist = np.sqrt(dx**2 + dy**2)

if method == 'full':

kernel = (1.0 / np.sqrt(-1j * self.lambda_val * dist)) * np.exp(-1j * self.k * dist)

elif method == 'phase_only_kernel':

kernel = self.phase_only_kernel(dist)

else:

kernel = (1.0 / np.sqrt(-1j * self.lambda_val * dist)) * np.exp(-1j * self.k * dist)

reconstructed_image[j, :] = kernel @ s

if j % 20 == 0:

print(f"    Progress: {j}/{len(self.y_i)}")

return reconstructed_image

def compute_fft_spectrum(self, image, out_size=512):

"""Compute 512x512 FFT spectrum"""

pad_y_total = out_size - image.shape[0]

pad_y_before = pad_y_total // 2

pad_y_after = pad_y_total - pad_y_before

pad_x_total = out_size - image.shape[1]

pad_x_before = pad_x_total // 2

pad_x_after = pad_x_total - pad_x_before

image_padded = np.pad(image, ((pad_y_before, pad_y_after),

(pad_x_before, pad_x_after)),

mode='constant')

return np.fft.fftshift(np.fft.fft2(image_padded))

def plot_results(self, reconstructed_image, fft_spectrum, sources, title_prefix, filename):

"""Plot reconstructed image and FFT spectrum"""

fig, axes = plt.subplots(1, 2, figsize=(16, 7))

ax1 = axes[0]

im1 = ax1.imshow(np.abs(reconstructed_image),

extent=[self.x_i.min(), self.x_i.max(), self.y_i.min(), self.y_i.max()],

cmap='viridis', origin='lower', aspect='auto')

ax1.set_title(f'{title_prefix}\nMagnitude of Reconstructed Image', fontsize=12)

ax1.set_xlabel('x (λ)')

ax1.set_ylabel('y (λ)')

plt.colorbar(im1, ax=ax1, label='Magnitude')

for (x_s, y_s) in sources:

ax1.plot(x_s, y_s, 'r*', markersize=10, mew=1, label='Source Location')

if len(sources) > 0:

ax1.legend(loc='upper right')

ax2 = axes[1]

im2 = ax2.imshow(np.log10(np.abs(fft_spectrum)**2 + 1e-9),

cmap='plasma', origin='lower')

ax2.set_title(f'{title_prefix}\n512×512 FFT Spectrum (log power)', fontsize=12)

ax2.set_xlabel('Spatial Frequency kx')

ax2.set_ylabel('Spatial Frequency ky')

plt.colorbar(im2, ax=ax2, label='log10(Power)')

plt.tight_layout()

plt.savefig(self.fig_dir / filename, dpi=150, bbox_inches='tight')

plt.close()

print(f"  Saved: {filename}")

def run_phase_only_analysis():

"""Run complete phase-only imaging analysis"""

imaging_system = PhaseOnlyImaging()

print("\n" + "="*70)

print("PHASE-ONLY IMAGING ANALYSIS")

print("="*70)

source_single = [(0, 0)]

sources_multiple = [

(0.0, 15 * imaging_system.lambda_val),

(-12 * imaging_system.lambda_val, -9 * imaging_system.lambda_val),

(0.0, -9 * imaging_system.lambda_val),

(12 * imaging_system.lambda_val, -9 * imaging_system.lambda_val),

]

print("\nGenerating original signals...")

s_A_original = imaging_system.simulate_received_signal(source_single)

s_B_original = imaging_system.simulate_received_signal(sources_multiple)

print("\n" + "="*70)

print("PART A: Phase-Only Received Signal")

print("="*70)

s_A_phase = imaging_system.phase_only_signal(s_A_original)

s_B_phase = imaging_system.phase_only_signal(s_B_original)

print("\nPart A - Single Source: Phase-only signal")

img_A_phase_signal = imaging_system.reconstruct_direct_superposition(s_A_phase, method='full')

fft_A_phase_signal = imaging_system.compute_fft_spectrum(img_A_phase_signal)

imaging_system.plot_results(img_A_phase_signal, fft_A_phase_signal, source_single,

'Part A: Single Source - Phase-Only Signal',

'partA_single_phase_signal.png')

print("\nPart A - Multiple Sources: Phase-only signal")

img_B_phase_signal = imaging_system.reconstruct_direct_superposition(s_B_phase, method='full')

fft_B_phase_signal = imaging_system.compute_fft_spectrum(img_B_phase_signal)

imaging_system.plot_results(img_B_phase_signal, fft_B_phase_signal, sources_multiple,

'Part A: Multiple Sources - Phase-Only Signal',

'partA_multiple_phase_signal.png')

print("\n" + "="*70)

print("PART B: Phase-Only Kernel")

print("="*70)

print("\nPart B - Single Source: Phase-only kernel")

img_A_phase_kernel = imaging_system.reconstruct_direct_superposition(s_A_original, method='phase_only_kernel')

fft_A_phase_kernel = imaging_system.compute_fft_spectrum(img_A_phase_kernel)

imaging_system.plot_results(img_A_phase_kernel, fft_A_phase_kernel, source_single,

'Part B: Single Source - Phase-Only Kernel',

'partB_single_phase_kernel.png')

print("\nPart B - Multiple Sources: Phase-only kernel")

img_B_phase_kernel = imaging_system.reconstruct_direct_superposition(s_B_original, method='phase_only_kernel')

fft_B_phase_kernel = imaging_system.compute_fft_spectrum(img_B_phase_kernel)

imaging_system.plot_results(img_B_phase_kernel, fft_B_phase_kernel, sources_multiple,

'Part B: Multiple Sources - Phase-Only Kernel',

'partB_multiple_phase_kernel.png')

print("\n" + "="*70)

print("PART B: Combined Phase-Only Signal and Kernel")

print("="*70)

print("\nPart B - Single Source: Phase-only signal + kernel")

img_A_phase_both = imaging_system.reconstruct_direct_superposition(s_A_phase, method='phase_only_kernel')

fft_A_phase_both = imaging_system.compute_fft_spectrum(img_A_phase_both)

imaging_system.plot_results(img_A_phase_both, fft_A_phase_both, source_single,

'Part B: Single Source - Phase-Only Both',

'partB_single_phase_both.png')

print("\nPart B - Multiple Sources: Phase-only signal + kernel")

img_B_phase_both = imaging_system.reconstruct_direct_superposition(s_B_phase, method='phase_only_kernel')

fft_B_phase_both = imaging_system.compute_fft_spectrum(img_B_phase_both)

imaging_system.plot_results(img_B_phase_both, fft_B_phase_both, sources_multiple,

'Part B: Multiple Sources - Phase-Only Both',

'partB_multiple_phase_both.png')

print(f"\nAll phase-only imaging results saved to '{imaging_system.fig_dir}' directory")

return {

'single_phase_signal': img_A_phase_signal,

'single_phase_kernel': img_A_phase_kernel,

'single_phase_both': img_A_phase_both,

'multiple_phase_signal': img_B_phase_signal,

'multiple_phase_kernel': img_B_phase_kernel,

'multiple_phase_both': img_B_phase_both

}

if __name__ == "__main__":

results = run_phase_only_analysis()